# RJA-40 Robotic Joint Actuator Module

Design Document  •  Rev. B  •  Ayr Robotics  •  Doc. No. DD-RJA40-B

# 1. Overview

The RJA-40 is an integrated brushless-motor joint actuator for collaborative robot arms, combining a BLDC motor, a two-stage planetary/harmonic drivetrain, an absolute encoder, and an onboard joint controller in a single module. It is designed for direct EtherCAT or CAN daisy-chaining across a 6- or 7-axis arm.

# 2. System Architecture

Figure 1 — RJA-40 drivetrain block diagram: motor, gearbox, harmonic drive stage, output flange, encoder feedback, and joint controller.

The absolute encoder reports output-flange position at 16-bit resolution to the joint controller, which closes a local current/velocity/position control loop at 4 kHz and exposes a CiA 402-style profile over EtherCAT or CAN to the arm's central controller.

# 3. Electrical & Mechanical Specifications

| Parameter | Value | Notes |
| --- | --- | --- |
| Rated torque | 40 N·m | continuous, output flange |
| Peak torque | 120 N·m | 1 s max |
| Gear ratio | 101:1 | planetary + harmonic stages |
| Rated output speed | 35 rpm | continuous |
| Operating voltage | 48 V DC | ±10% |
| Continuous current | 8 A | bus current |
| Peak current | 22 A | 1 s max |
| Encoder resolution | 16-bit absolute | output side |
| Backlash | < 3 arcmin | measured at output flange |
| Operating temperature | 0 °C to 50 °C | housing surface |
| Weight | 1.35 kg | module only |
| Ingress protection | IP54 | shaft seal fitted |

Figure 2 — RJA-40 module, assembled view.

# 4. Known Limitations

The Rev. B module does not include an integrated torque sensor at the output flange; torque is estimated from motor current, which is less accurate under high-friction or high-backlash conditions. A strain-gauge-based torque sensor is planned for Rev. C.